Referenced File: Input_Adaptation_01.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_01_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Adaptation Components | 幻灯片标题： 配接部件 |
| Slide Main Text: Input adaptation components connect the transmission torque converter to the engine crankshaft. Front-end transmission component damage is often related to input adaptation. Visually inspect all adaptation components for wear, damage or distortion. Perform input adaptation measurements prior to installing the transmission. Input adaptation reference resources. The Mechanic’s Tips booklets. Service Information Letter (SIL) 60-TR-81 (available on the Allison Extranet). | 幻灯片正文： 输入配接部件将变速箱扭矩变换器连接到引擎曲轴。 前端变速箱部件损坏通常与输入配接有关。 视觉检查所有配接部件，看有无磨损、损坏或扭曲。 在安装变速箱之前，应进行输入配接测量。 输入配接参考资源。 技师提示小册子。 维修信函（SIL）60-TR-81（可从阿里逊外部网上获取）。 |

Referenced File: Input_Adaptation_02.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_02_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Measuring Tools | 幻灯片标题： 测量工具 |
| Slide Main Text: Required Measuring Tools 600mm (24 inch) vernier caliper. 50-100mm (2-4 inch) telescoping gauge. 25-76mm (1-3 inch) outside micrometer. Dial indicator and mounting attachments. 0-150mm (0-6 inch) depth micrometer. Clean and inspect all components thoroughly prior to performing input adaptation measurements. When installing crankshaft bolts for measuring purposes, torque the bolts per the engine manufacturer’s recommendations. | 幻灯片正文： 需要的测量工具 600mm (24 inch) 游标卡尺。 50-100mm (2-4 英寸) 伸缩规。 25-76mm (1-3 英寸) 外径千分尺。 刻度盘指示器和安装附件。 0-150mm (0-6 英寸) 深度千分尺。 在进行输入配接测量之前，彻底清洁并检查所有部件。 当为测量目的安装曲轴螺栓时，将螺栓拧紧到引擎制造商建议的扭矩规格。 |

Referenced File: Input_Adaptation_03.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_03_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Flywheel Housing Pilot Bore Diameter | 幻灯片标题： 飞轮罩导孔直径 |
| Slide Main Text: Use an inside micrometer or vernier caliper. Measure the housing where the transmission torque converter housing lip rests. | 幻灯片正文： 利用一只内径千分尺或游标卡尺测量。 测量变速箱扭矩变换器外壳舌安装处的飞轮罩。 |

Referenced File: Input_Adaptation_04.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_04_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Flywheel Housing Bore Runout | 幻灯片标题： 飞轮罩孔偏摆 |
| Slide Main Text: Mount the dial indicator base on the crankshaft hub. Position the dial indicator button on the inside diameter of the flywheel housing. Zero the dial indicator. Rotate the crankshaft one complete revolution. Read Total Indicated Runout (TIR) – all indicator movement both positive and negative. Runout can sometimes be changed by repositioning the flywheel housing on the back of the engine. | 幻灯片正文： 将刻度盘指示器座安装在曲轴轮毂上。 将刻度盘指示器按钮放在飞轮罩内径上。 将刻度盘指示器调零。 旋转曲轴一周。 读出总指示偏摆（TIR）- 所有指示器移动，包括正负移动。 有时候，可改变飞轮罩在引擎背后的位置而改变偏摆。 |

Referenced File: Input_Adaptation_05.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_05_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Flywheel Housing Face Squareness | 幻灯片标题： 飞轮罩面正方度 |
| Slide Main Text: Mount the dial indicator base on the crankshaft hub. Position the dial indicator button on the face of the flywheel housing. Zero the dial indicator. Rotate the crankshaft one complete revolution. Read Total Indicated Runout (TIR). | 幻灯片正文： 将刻度盘指示器座安装在曲轴轮毂上。 将刻度盘指示器按钮放在飞轮罩面上。 将刻度盘指示器调零。 旋转曲轴一周。 读出总指示偏摆（TIR）。 |

Referenced File: Input_Adaptation_06.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_06_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Crankshaft Hub Pilot or Adapter Diameter | 幻灯片标题： 曲轴轮毂导孔配接器直径 |
| Slide Main Text: Measure using an inside micrometer or a telescoping gauge and outside micrometer. Take measurements in at least two locations. | 幻灯片正文： 使用一只内径千分尺或伸缩规和外径千分尺测量。 至少在两个位置进行测量。 |

Referenced File: Input_Adaptation_07.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_07_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Crankshaft Hub Pilot or Adapter Face Squareness | 幻灯片标题： 曲轴轮毂导孔或配接器面正方度 |
| Slide Main Text: Install the hub adapter if used – torque bolts to engine manufacturer’s specification. Mount the dial indicator base on the flywheel housing. Position the dial indicator button on the face of the hub pilot or adapter. Zero the dial indicator. Rotate the crankshaft one complete revolution. Read Total Indicated Runout (TIR). | 幻灯片正文： 安装轮毂配接器，如使用的话 – 将螺栓拧紧到引擎制造商的扭矩规格。 将刻度盘指示器座安装在飞轮罩上。 将刻度盘指示器按钮放在轮毂导孔或配接器面上。 将刻度盘指示器调零。 旋转曲轴一周。 读出总指示偏摆（TIR）。 |

Referenced File: Input_Adaptation_08.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_08_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Crankshaft Hub Pilot or Adapter Concentricity | 幻灯片标题： 曲轴轮毂导孔配接器同心性 |
| Slide Main Text: Install the hub adapter if used – torque bolts to engine manufacturer’s specification. Mount the dial indicator base on the flywheel housing. Position the dial indicator button on the inside bore of the hub pilot or adapter. Zero the dial indicator. Rotate the crankshaft one complete revolution. Read Total Indicated Rounout (TIR). | 幻灯片正文： 安装轮毂配接器，如使用的话 – 将螺栓拧紧到引擎制造商的扭矩规格。 将刻度盘指示器座安装在飞轮罩上。 将刻度盘指示器按钮放在轮毂导孔或配接器内孔上。 将刻度盘指示器调零。 旋转曲轴一周。 读出总指示偏摆（TIR）。 |

Referenced File: Input_Adaptation_09.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_09_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Flexplate bolt hole flatness | 幻灯片标题： 挠性板螺栓孔平整度 |
| Slide Main Text: Install the hub adapter, flexplates, flexplate adapter and wear plate on the engine and torque the crankshaft bolts. Mount the dial indicator base on the flywheel housing. Position the dial indicator button on the flexplates 292mm (11.5 inches) from the center of the crankshaft. Zero the dial indicator. Rotate the crankshaft one complete revolution. Read Total Indicated Runout (TIR). | 幻灯片正文： 将轮毂配接器、挠性板、挠性板配接器和耐磨板安装在引擎上并将曲轴螺栓拧紧至扭矩规格。 将刻度盘指示器座安装在飞轮罩上。 将刻度盘指示器按钮放在挠性板上距曲轴中心292mm (11.5英寸)处。 将刻度盘指示器调零。 旋转曲轴一周。 读出总指示偏摆（TIR）。 |

Referenced File: Input_Adaptation_10.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_10_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Torque Converter Axial Location | 幻灯片标题： 扭矩变换器轴位置 |
| Slide Main Text: 1000 & 2000 Product Families Install the hub adapter, flexplates and wear plate and torque the bolts. Install the flexplate adapter to the flexplates and torque the bolts. Measure from the face of the flywheel housing to the face of the flexplate adapter at 292.1mm (11.5 inches) from the center of the crankshaft. | 幻灯片正文： 1000和2000产品家族 安装轮毂配接器、挠性板和耐磨板，并将螺栓拧紧至扭矩规格。 将挠性板配接器安装到挠性板上，并将螺栓拧紧至扭矩规格。 从飞轮罩表面到挠性板配接器表面距曲轴中心292.1mm (11.5 英寸)处测量。 |

Referenced File: Input_Adaptation_11.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_11_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Torque Converter Axial Location | 幻灯片标题： 扭矩变换器轴位置 |
| Slide Main Text: 3000 & 4000 Product Families Remove the flexplate adapter from the torque converter. Position the transmission output shaft down, torque converter installed. Use a depth gauge to measure from the face of the torque converter housing to the torque converter flexplate adapter mounting surface. | 幻灯片正文： 3000和4000产品家族 从扭矩变换器上拆下挠性板配接器。 将变速箱输出轴置于向下位置，并安装扭矩变换器。 利用一只深度规从扭矩变换器外壳的表面到扭矩变换器挠性板配接器安装表面进行测量。 |

Referenced File: Input_Adaptation_12.fla

Type: Main Text

Text and Voice Over

Voice Over Reference File: Input_Adaptation_12_VO.mp3

| Module Title: Input Adaptation | 单元标题： 输入配接 |
| --- | --- |
| Slide Header: Final Component Installation | 幻灯片标题： 最后的部件安装 |
| Slide Main Text: Position the crankshaft hub adapter on the back of the engine. Position the flexplates on the crankshaft hub adapter. Position the wear plate on the installed flexplates. Install and torque the bolts securing the entire assembly to the back of the engine. Install the flexplate adapter to the flexplates or torque converter (depending on configuration) and torque the mounting bolts. | 幻灯片正文： 将曲轴轮毂配接器置于引擎的背面。 将挠性板置于曲轴轮毂配接器上。 将耐磨板置于安装的挠性板上。 安装螺栓并拧紧至扭矩规格，将整个组件固定到引擎的背面。 将挠性板配接器安装到挠性板上或扭矩变换器上（取决于配置）并将安装螺栓拧紧至扭矩规格。 |