Referenced File: Input_Adaptation_01.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Typical Adaptation Hardware Components Flexplate Adapter Wearplate Crankshaft Hub Adapter Transmission Flexplate Assembly Starter Ring Gear Flywheel Housing Engine | Callout01- 典型的配接硬件部件 挠性板配接件 耐磨板 曲轴轮毂配接器 变速箱 挠性板组件 起动器齿圈 飞轮罩 引擎 |
| --- | --- |

Referenced File: Input_Adaptation_02.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Precision Caliper Micrometer Telescoping Gauge Mounting Attachments Dial Indicator | Callout01- 精确卡尺 千分尺 伸缩规 安装附件 刻度盘指示器 |
| --- | --- |

Referenced File: Input_Adaptation_03.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Flywheel Housing Bore Diameter Product Families Specifications 1000 & 2000 SAE 3 Housing 409.58-409.70 mm (16.125-16.130 inches) 1000 & 2000 SAE 2 Housing 447.68-447.80 mm (17.625-17.630 inches) 3000 Product Family 447.68-447.80 mm (17.625-17.630 inches) 4000 Product Family 511.18-511.30 mm (20.125-20.130 inches) | Callout01- 飞轮罩孔径 产品家族 规格 1000和2000 SAE 3 飞轮罩 409.58-409.70 mm (16.125-16.130 英寸) 1000和2000 SAE 2 飞轮罩 447.68-447.80 mm (17.625-16.630 英寸) 3000产品家族 447.68-447.80 mm (17.625-17.630 英寸) 4000产品家族 511.18-511.30 mm (20.125-20.130 英寸) |
| --- | --- |
| Callout02- The measuring instrument should intersect the housing centerline. | Callout02- 测量仪器应该与飞轮罩中心线相交。 |

Referenced File: Input_Adaptation_04.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Position the indicator button on the housing inside diameter. | Callout01- 将指示器按钮放在飞轮罩内径上。 |
| --- | --- |
| Callout02- Flywheel Housing Bore Runout Product Families Specifications 1000, 2000, 3000 & 4000 Product Families Cannot exceed 0.51 mm (0.020 inches) TIR | Callout02- 飞轮罩孔偏摆 产品家族 规格 1000、2000、3000和4000产品家族 不能超过0.51 mm (0.020 英寸) TIR |

Referenced File: Input_Adaptation_05.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Position the indicator button on the housing face. Avoid allowing the button to contact bolt holes. | Callout01- 将指示器按钮放在飞轮罩面上。 避免按钮接触螺栓孔。 |
| --- | --- |
| Callout02- Flywheel Housing Face Squareness Product Families Specifications 1000, 2000, 3000 & 4000 Product Families Cannot exceed 0.51 mm (0.020 inches) TIR | Callout02- 飞轮罩面正方度 产品家族 规格 1000、2000、3000和4000产品家族 不能超过0.51 mm (0.020 英寸) TIR |

Referenced File: Input_Adaptation_06.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Place gauge in bore’s centerline. | Callout01- 将量规放在孔径中心线上。 |
| --- | --- |
| Callout02- Crankshaft Hub Pilot Diameter Product Families Specifications 1000 & 2000 Product Families 43.26-43.32 mm (1.703-1.705 inches) 3000 & 4000 Product Families 50.94-50.99 mm (2.006-2.008 inches) | Callout02- 曲轴轮毂导孔直径 产品家族 规格 1000和2000产品家族 43.26-43.32 mm (1.703-1.705 英寸) 3000和4000产品家族 50.94-50.99 mm (2.006-2.008 英寸) |

Referenced File: Input_Adaptation_07.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Torque crankshaft bolts to engine manufacturer’s specification. | Callout01- 将曲轴螺栓拧紧到引擎制造商的扭矩规格。 |
| --- | --- |
| Callout02- Crankshaft Hub Adapter Face Squareness Product Families Specifications 1000, 2000, 3000 & 4000 Product Families Cannot exceed 0.013 mm (0.0005 inches) TIR | Callout02- 曲轴轮毂配接器表面正方度 产品家族 规格 1000、2000、3000和4000产品家族 不能超过0.013 mm (0.0005 英寸) TIR |

Referenced File: Input_Adaptation_08.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Position the indicator button or extension on the inside of the bore. | Callout01- 将指示器按钮或延展放在孔内。 |
| --- | --- |
| Callout02- Crankshaft Hub Pilot Concentricity Product Families Specifications 1000 & 2000 Product Families Cannot exceed 0.25 mm (0.010 inches) TIR 3000 & 4000 Product Families Cannot exceed 0.13 mm (0.005 inches) TIR | Callout02- 曲轴轮毂导孔同心度 产品家族 规格 1000和2000产品家族 不能超过0.25 mm (0.010英寸) TIR 3000和4000产品家族 不能超过0.13 mm (0.005 英寸) TIR |

Referenced File: Input_Adaptation_09.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Flexplate | Callout01- 挠性板 |
| --- | --- |
| Callout02- Avoid reading crankshaft end play. | Callout02- 避免读取曲轴轴端余隙 |
| Callout03- Flexplate Bolt Hole Flatness Product Families Specifications 1000 & 2000 Product Families Cannot exceed 0.76 mm (0.030 inch) TIR when measured at 292 mm (11.5 inches) from the center of the crankshaft 3000 & 4000 Product Families Measurement Does Not Apply | Callout03- 挠性板螺栓孔平整度 产品家族 规格 1000和2000产品家族 当从曲轴中心292 mm (11.5英寸)处测量时，不能超过0.76 mm (0.030英寸) TIR 3000和4000产品家族 测量不适用 |

Referenced File: Input_Adaptation_10.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Flywheel Housing | Callout01- 飞轮罩 |
| --- | --- |
| Callout02- Flexplate | Callout02- 挠性板 |
| Callout03- Use a depth gauge to measure. Make sure the crankshaft is thrust all the way rearward. | Callout03- 使用深度规测量。 确保曲轴一直伸到最后。 |
| Callout04- Converter Axial Location Product Families Specifications 1000 & 2000 SAE 3 Housing 40.15-44.21 mm (1.581-1.741 inches) 1000 & 2000 SAE 2 Housing 30.50-34.56 mm (1.201-1.361 inches) | Callout04- 变换器轴位置 产品家族 规格 1000和2000 SAE 3 飞轮罩 40.15-44.21 mm (1.581-1.741英寸) 1000和2000 SAE 2 飞轮罩 30.50-34.56 mm (1.201-1.361英寸) |

Referenced File: Input_Adaptation_11.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Converter Axial Location Product Families Specifications 3000 Product Family 49.36-50.38 mm (1.943-1.983 inches) 4000 Product Family 45.54 mm (1.793 inches) | Callout01- 变换器轴位置 产品家族 规格 3000产品家族 49.36-50.38 mm (1.943-1.983英寸) 4000产品家族 45.54 mm (1.793英寸) |
| --- | --- |
| Callout02- In this example, the straight-edge width is subtracted from the depth gauge reading. | Callout02- 本例中，直边宽度从深度规读数中减去。 |

Referenced File: Input_Adaptation_12.fla

Type: Main Layout

Text Only (unless it has been voiced then note)

| Callout01- Starter Ring Gear Flexplate Crankshaft Transmission Main Housing Flexplate Adapter Converter Drive Cover Wearplate Crankshaft Hub Adapter Converter Pilot | Callout01- 起动器齿圈 挠性板 曲轴 变速箱主机箱 挠性板配接件 变换器驱动盖 耐磨板 曲轴轮毂配接器 变换器导孔 |
| --- | --- |
| Callout02- Typical adaptation hardware installation. | Callout02- 典型配接硬件安装 |