Referenced File: nav.xml

Type: Q&A Text

***Text Only***

| Quiz Title: Input Adaptation | 测验标题： 输入配接 |
| --- | --- |
| Question 01 Input adaptation components: Must be measured according to the engine manufacturer’s guidelines – related support and technical information is not available from Allison Transmission. Are required only when installing Allison transmissions to Cummins engines. Connect the transmission torque converter to the engine crankshaft and must be inspected and measured prior to final installation. | 问题01 输入配接部件： 必须按照引擎制造商的准则进行测量 – 阿里逊变速箱不提供相关的支持和技术信息。 仅在将阿里逊变速箱安装到Cummins引擎上时才需要。 将变速箱扭矩变换器连接到引擎曲轴上，在最后安装之前，必须检查并测量。 |

| Question 02 Input adaptation measuring tools: Are only available from Allison Distributors and Dealers. Include dial indicators and micrometers. Must be re-purchased whenever input adaptation measurements are performed because the tools are left installed on the vehicle. | 问题02 输入配接测量工具： 仅可从阿里逊分销商或经销商处获得。 包括刻度盘指示器和千分尺。 任何时候进行输入配接测量时都需要重新购买，因为工具保留安装在车辆上。 |
| --- | --- |

| Question 03 Flywheel housing pilot bore diameter: Is measured on the flywheel housing where the transmission torque converter housing lip rests. Must be measured using an outside micrometer. Requires that the transmission must be temporarily installed to the back of the engine. | 问题03 飞轮罩导孔直径： 是在飞轮罩上变速箱扭矩变换器外壳舌所处的位置测量的。 必须用一只外径千分尺测量。 要求变速箱必须暂时安装到引擎的背面。 |
| --- | --- |

| Question 04 Flywheel housing bore runout: Requires that the dial indicator base is installed to the flywheel housing. Indicates the flywheel housing position on the back of the engine and cannot be changed or adjusted. Is 0.005 inches TIR when the indicator travels 0.002 inches positive and 0.003 inches negative during one crankshaft revolution. | 问题04 飞轮罩孔偏摆： 要求刻度盘指示器座安装到飞轮罩上。 指明飞轮罩在引擎背面的位置，且不能改变或调整。 在曲轴转动一周过程中当指示器正向移动0.002 英寸、反向移动0.003英寸时，为0.005英寸TIR。 |
| --- | --- |

| Question 05 Flywheel housing face squareness: Requires that the dial indicator button is positioned on the back of the crankshaft. Is determined by reading positive dial indicator movement only, not TIR. Is measured with the dial indicator button positioned on the face of the flywheel housing. | 问题05 飞轮罩面正方度： 要求刻度盘指示器按钮位于曲轴的背面。 通过读取刻度盘指示器正向移动，而不是TIR确定的。 是在刻度盘指示器按钮位于飞轮罩面上时测量的。 |
| --- | --- |

| Question 06 Crankshaft hub adapter pilot diameter: Is measured using an inside micrometer or a telescoping gauge and outside micrometer. Should be measured in one location on the pilot only. Requires a telescoping gauge placed in an off-center location in the pilot bore. | 问题06 曲轴轮毂配接器导孔直径： 是用一只内径千分尺或伸缩规和外径千分尺测量的。 应该仅在导孔上一个位置测量。 要求伸缩规放置在导孔上的一个偏心位置。 |
| --- | --- |

| Question 07 Crankshaft hub adapter face squareness: Is measured with the dial indicator base mounted to the transmission torque converter housing. Requires that the hub adapter is installed to the engine crankshaft. Can be measured with a single bolt installed to the crankshaft and that bolt does not need to be torqued. | 问题07 曲轴轮毂配接器表面正方度： 是在刻度盘指示器座安装到变速箱扭矩变换器外壳上时测量的。 要求轮毂配接器安装到引擎的曲轴上。 可在用一只螺栓安装到曲轴上且该螺栓并不需要拧紧到扭矩规格的情况下进行测量。 |
| --- | --- |

| Question 08 Crankshaft hub adapter concentricity: Is not a required input adaptation component measurement. Requires mounting the dial indicator base to one of the crankshaft bolts. Is measured by positioning the dial indicator button or extension on the inside of the hub adapter bore. | 问题08 曲轴轮毂配接器同心度： 不是一个要求的输入配接部件测量。 要求将刻度盘指示器座安装到一个曲轴螺栓上。 是通过将刻度盘指示器按钮或延展放在轮毂导孔配接器内孔上测量的。 |
| --- | --- |

| Question 09 Flexplate bolt hole flatness: Requires temporary installation of the hub adapter, flexplates and wear plate. Is not affected by crankshaft end play when measuring. Is not a required input adaptation component measurement. | 问题09 挠性板螺栓孔平整度： 要求临时安装轮毂配接器、挠性板和耐磨板。 测量时不受曲轴轴端余隙的影响。 不是一个要求的输入配接部件测量。 |
| --- | --- |

| Question 10 1000 & 2000 Product Families torque converter axial location: Is measured as close to the center of the flexplate as possible. Must be measured with the flexplates removed from the back of the engine. Is measured from the face of the flywheel housing to the face of the flexplate adapter at 11.50 inches from the center of the crankshaft. | 问题10 1000 和2000产品家族扭矩变换器轴位置： 尽量测量到挠性板的中心。 测量时必须从引擎背面拆除挠性板。 从飞轮罩表面到挠性板配接器表面距曲轴中心11.50英寸 处测量。 |
| --- | --- |

| Question 11 3000 & 4000 Product Families torque converter axial location: Is measured using the same procedures as 1000 & 2000 Product Families. Can be measured with the transmission removed, positioned output shaft down, with the torque converter installed. Requires the use of a telescoping gauge to perform the measurement. | 问题11 3000 和4000产品家族扭矩变换器轴位置： 采用与1000和2000产品家族同样的程序测量。 可在卸除了变速箱、输出轴定位向下以及扭矩变换器安装了的情况下进行测量。 需要使用伸缩规进行测量。 |
| --- | --- |

| Question 12 Final input adaptation component installation: Must be done prior to performing any input adaptation component measurements. Requires that the flexplates be installed to the back of the crankshaft before installing the crankshaft hub adapter. Includes positioning the hub adapter, flexplates and wear plate on the back of the engine and installing and torquing the crankshaft bolts to specification. | 问题12 输入配接部件最终安装： 必须在进行任何输入配接部件测量之前完成。 在安装曲轴轮毂配接器之前，要求将挠性板安装到曲轴的背面。 包括将轮毂配接器、挠性板和耐磨板在引擎背面定位，以及安装曲轴螺栓并拧紧至扭矩规格。 |
| --- | --- |