Referenced File: Input_Adaptation_01.fla

Type: Resource Center

| Title (Text Only) Introduction Video | 标题（仅限文字） 介绍视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Overview.flv) Adaptation hardware properly connects the transmission torque converter to the engine crankshaft. These components usually include the crankshaft hub adapter, the flex plates and the wear plate. The crankshaft hub adapter is installed on the back of the engine crankshaft. The chamfered outer edge of the hub adapter always faces toward the transmission. The flex plates are stacked against the hub adapter, and the wear plate is next. The entire assembly is secured by bolts that run through each component’s holes and thread into the back of the crankshaft. | 内容（画外音）(3K_Adaptation Overview.flv) 配接硬件将变速箱扭矩变换器适当地连接到引擎曲轴。 这些部件通常包括曲轴轮毂配接器、挠性板和耐磨板。 将曲轴轮毂配接器安装在引擎曲轴的背后。 轮毂配接器带倒棱的外缘总是面向变速箱。 挠性板叠加在轮毂配接器上，耐磨板为下一个。 整个组件用穿过每个部件中的孔的螺栓固定，并拧到曲轴背后。 |
| Title (Text Only) 1000 & 2000 Mechanic’s Tips Download Demonstration | 标题（仅限文字） 1000和2000技师提示下载示范 |
| Content No Translation, video content only (1kadapfinal.flv) | 内容 No Translation, video content only (1kadapfinal.flv) |
| Title (Text Only) 3000 & 4000 Mechanic’s Tips Download Demonstration | 标题（仅限文字） 3000和4000技师提示下载示范 |
| Content No Translation, video content only (3kadapfinal.flv) | Content No Translation, video content only (3kadapfinal.flv) |
| Title (Text Only) SIL 60-TR-81 Download Demonstration | 标题（仅限文字） SIL 60-TR-81下载示范 |
| Content No Translation, video content only (siladapfinal.flv) | Content No Translation, video content only (siladapfinal.flv) |

Referenced File: Input_Adaptation_02.fla

Type: Resource Center

NO TRANSLATION

Referenced File: Input_Adaptation_03.fla

Type: Resource Center

| Title (Text Only) Bore Diameter Video | 标题（仅限文字） 孔径视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Fly House Bore Diam.flv) Use an inside micrometer or vernier caliper to measure flywheel housing inside bore diameter. Make sure the measuring instrument intersects the housing centerline. | 内容（画外音）(3K_Adaptation Fly House Bore Diam.flv) 利用内径千分尺或游标卡尺测量飞轮罩内孔径。 确保测量仪器与飞轮罩中心线交叉。 |

Referenced File: Input_Adaptation_04.fla

Type: Resource Center

| Title (Text Only) Bore Runout Video | 标题（仅限文字） 孔振摆视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Fly House Bore Eccentricity.flv) Mount the dial indicator base on the crankshaft hub, and position the indicator’s button on the inside diameter of the flywheel housing. Zero the indicator and rotate the crankshaft one complete revolution. Total Indicated Runout, or TIR, cannot exceed twenty-thousandths of an inch or .51 millimeters. Remember, TIR is the sum total of all indicator movement. If the indicator moves five thousandths negative, then eight thousandths positive, TIR is thirteen thousandths. | 内容（画外音）(3K_Adaptation Fly House Bore Eccentricity.flv) 将刻度盘指示器座安装在曲轴轮毂上，将指示器按钮置于飞轮罩的内径上。 将指示器调零并转动曲轴一周。 总指示振摆或TIR不得超过千分之二十英寸，或0.51毫米。 记住，TIR是指示器所有移动的总和。 如果指示器负向移动千分之五，然后正向移动千分之八，则TIR为千分之十三。 |

Referenced File: Input_Adaptation_05.fla

Type: Resource Center

| Title (Text Only) Face Squareness Video | 标题（仅限文字） 表面正方度视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Fly House Face Squareness.flv) The flywheel housing face must be checked for squareness. Keep the indicator base mounted on the crankshaft hub, but move the indicator button to the face of the housing where the torque converter housing mates. Position the indicator so the button avoids the bolt holes. Then zero the indicator and rotate the crankshaft one complete revolution. Total all indicator movement to determine TIR. Face squareness cannot exceed twenty thousandths of an inch or .51 millimeters. A good rule of thumb to use on all measurements is to check your indicator reading at the end of one crankshaft revolution. If the indicator hasn’t returned to zero, its mount might not be stable, or your readings might be affected by crankshaft endplay. If this occurs, reset the indicator to zero then perform the check again. | 内容（画外音）(3K_Adaptation Fly House Face Squareness.flv) 必须检查飞轮罩表面的正方度。 保持指示器座安装在曲轴轮毂上，但是将指示器按钮移到飞轮罩表面与扭矩变换器对接的地方。 将指示器置于这样的位置，使得按钮避开螺栓孔。 然后将指示器调零并转动曲轴一周。 计算指示器的总移动以确定TIR。表面正方度不得超过千分之二十英寸或0.51毫米。 一个很好的可用于所有测量的经验法则就是在曲轴转动一周结束时检查指示器的读数。 如果指示器没有回到零，则这一安装可能不稳定，或者读数可能受到了曲轴轴端间隙的影响。 如果发生这种情况，重置指示器使之归零，然后再进行检查。 |

Referenced File: Input_Adaptation_06.fla

Type: Resource Center

| Title (Text Only) Hub Pilot Diameter Video | 标题（仅限文字） 轮毂导孔直径视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Hub Adapt In Pilot Diameter.flv) Place a telescoping gauge in the bore’s centerline. Lock the gauge and measure its length with an outside micrometer. Compare your measurement to specifications. | 内容（画外音）(K_Adaptation Hub Adapt In Pilot Diameter.flv) 将伸缩规放在孔中心线上。锁定伸缩规并用一外径千分尺测量其长度。 将测量结果与规格比较。 |

Referenced File: Input_Adaptation_07.fla

Type: Resource Center

| Title (Text Only) Hub Face Squareness Video | 标题（仅限文字） 轮毂表面正方度视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Hub Adapt Face Squareness.flv) The hub adapter’s face must be checked for squareness. Mount a dial indicator as shown to measure Total Indicated Runout, or TIR. TIR is determined by totaling all indicator movement. For instance, if the indicator moves two thousandths to the negative then three thousandths to the positive, this represents five thousandths Total Indicated Runout. When measuring hub face squareness, be sure to remove crankshaft end play. Face squareness measurements will be inaccurate if the crankshaft is floating back and forth. Keep steady forward or backward pressure on the crankshaft as it’s rotated. | 内容（画外音）(3K_Adaptation Hub Adapt Face Squareness.flv) 必须检查轮毂配接器表面的正方度。 将刻度盘指示器按所示的方法安装以测量总指示振摆，或TIR。振摆是通过对所有指示器移动求和确定的。 例如，如果指示器负向移动千分之二，然后正向移动千分之三，这代表总指示振摆为千分之五。 当测量轮毂面正方度时，确保要去掉曲轴轴端间隙。 如果曲轴前后漂浮，表面正方度测量将会不准确。 曲轴转动时，保持稳定正向和反向压力。 |

Referenced File: Input_Adaptation_08.fla

Type: Resource Center

| Title (Text Only) Hub Pilot Concentricity Video | 标题（仅限文字） 轮毂导孔同心度视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Hub Adapt Bore Eccentricity.flv) Mount a swivel attachment on the dial indicator and place the end of the swivel so it’s touching the crankshaft hub pilot bore. Zero the dial indicator and slowly rotate the crankshaft exactly on revolution. | 内容（画外音）(3K_Adaptation Hub Adapt Bore Eccentricity.flv) 在刻度盘指示器上安装一旋转附件，放置旋转附件的端点使之接触曲轴轮毂导孔。 将刻度盘指示器调零并缓慢地转动曲轴准确一周。 |

Referenced File: Input_Adaptation_09.fla

Type: Resource Center

| Title (Text Only) Flexplate Flatness Video | 标题（仅限文字） 挠性板平整度视频 |
| --- | --- |
| Content (Voice Over) (1K_Adaptation Flexplate RemountFlat.flv) We need to mount both the flexplate and the flexplate adapter back onto the crankshaft hub adapter, and then measure the flatness at the bolt circle where the torque converter attaches. Always make sure that the final tightening of the crankshaft bolts is to the manufacturer’s specification. For measuring the flatness of the flexplate, we’re going to mount a dial indicator base on the flywheel housing with the pointer on the flexplate just outside of the bolt circle where the torque converter would mate up. We want to make sure we check the bolt holes for burrs or anything that would give a false reading. Ok, the flexplate is in good shape. | 内容（画外音）(1K_Adaptation Flexplate RemountFlat.flv) 我们需要将挠性板和挠性板配接器装回到曲轴轮毂配接器上，然后在扭矩变换器安装的螺栓圈处测量平整度。 始终要确保最后将曲轴螺栓拧紧到制造商的扭矩规格。 测量挠性板的平整度的时候，要在飞轮罩上安装一只刻度盘指示器座，指针位于挠性板上螺栓圈外面一点与扭矩变换器对接的地方。 我们要确保检查螺栓孔有无毛刺或任何可能产生虚假读数的情况。好的，挠性板处于良好状态。 |

Referenced File: Input_Adaptation_10.fla

Type: Resource Center

| Title (Text Only) Converter Axial Location Video | 标题（仅限文字） 变换器轴位置视频 |
| --- | --- |
| Content (Voice Over) (1K_Adaptation Axial Location.flv) We want to make sure that the flexplate is in the right axial location so that the torque converter is properly located. We’re going to mount a straight edge on the face of the flywheel housing. Then we’re going to measure from the face of the straight edge down to the flexplate. Ok, if we subtract the one inch thickness of our straight edge, that puts us right in specifications. Another important thing to remember when checking the flexplate axial location is before you start, make sure you’ve moved the crankshaft all the way to the rear to take out any crankshaft endplay. | 内容（画外音）(1K_Adaptation Axial Location.flv) 我们要确保挠性板处于正确的轴位置，以便扭矩变换器能够适当定位。 我们要在飞轮罩的表面上安装一个直边。 然后，我们将从直边的表面上向下量到挠性板。 好的，如果我们将直边的厚度减去1英寸，那就符合规格了。 检查组挠性板轴位置的时候，另一个重要的事项是要记住，在开始之前，要确保将曲轴移到最后面，以便消除任何曲轴轴端间隙。 |

Referenced File: Input_Adaptation_11.fla

Type: Resource Center

| Title (Text Only) Converter Axial Location Video | 标题（仅限文字） 变换器轴位置视频 |
| --- | --- |
| Content (Voice Over) (3K_Adaptation Converter Axial Position.flv) Converter axial position is measured with the flexplate adapter removed. Place the transmission on a stand and position it with the converter straight up. Then use a depth gauge to perform the measurement. | 内容（画外音）(3K_Adaptation Converter Axial Position.flv) 变换器轴位置是在卸下挠性板配接器的情况下测量的。 将变速箱放在台上并使变换器处于直立位置。 然后用深度规进行测量。 |

Referenced File: Input_Adaptation_12.fla

Type: Resource Center

NO TRANSLATION