### Frame 1-

Header:

Overview

Main Text:

For approximately two years following TC10 Start of Production (SOP), service procedures will mainly consist of transmission Removal and Replacement (R&R).

This will allow Allison Service Engineers to gather data on operational issues that may arise during initial product deployment

This will allow the development of field service procedures that will then be released to the service channel during the initial two-year evaluation period

During this time frame, several maintenance level repairs have been developed for the service channel that may be performed prior to/in-lieu of a full transmission R&R procedure.

Maintenance level repairs for the TC10 are defined the same way as other Allison

Transmisison products, meaning the repairs cannot involve a “split-line” separation

Each procedure will be released to the service channel in a SIL which will contain a detailed description of the procedure, required parts list and torque values where applicable

This module will provide a brief overview of those procedures.

Graphics:

Glamour shot of TC10.  Rotate through TC10 preventive maintenance stills (from Jan.14 shoot)

Links:

None

Frame 2-

Header:

Special Tools

Main Text:

Special Tools required for TC10 maintenance level repairs include:

J 37034 – Seal installer handle

J 37032 – Input seal installer

J 37031-A – Output seal installer

J 51170 – Reverse idler gear holding fixture

J 51026 – Lifting bracket

J 51056 – Shift-fork sensor alignment tool

J 51015 – TC10 Reflash harness

J 51058 – TC10 external TCM test harness

Graphics:

Rotating line art of special tools (if available), or stills from TC10 PM shoot

Links:

Interactive image of special tools arranged on bench.  Rollover each tool to show larger image and descriptive text.

Frame 3-

Header:

In-Chassis Procedures

Main Text:

In-chassis repair procedures include:

TCM R&R

Cooler return filter and housing R&R

Output speed sensor R&R

Oil pan R&R

Output seal R&R

Graphics:

Rotating images highlighting each procedure as it is read (taken from TC10 PM shoot)

Links:

TCM R&R video

Output speed sensor R&R video

Cooler return filter and housing R&R video

Output speed sensor R&R video

Oil pan R&R video

Output seal R&R video

Frame 4-

Header:

In-Chassis Procedures (cont’d)

Main Text:

Additional in-chassis repair procedures include:

Control valve module component replacement, including:

Turbine speed sensor

Range output speed sensor

Linear Position Sensor (LPS)

Solenoids

Pressure switch manifolds

Internal wiring harness

Oil Level Sensor (OLS)

Shift fork magnet carrier R&R

Control module R&R

Shift selector R&R

Graphics:

Rotating images highlighting each procedure as it is read (taken from TC10 PM shoot)

Links:

Turbine, range output and LPS sensors R&R video

Solenoid R&R video

Pressure switch manifolds R&R video

Internal wiring harness R&R video

OLS R&R video

Shift fork magnet carrier R&R video

Control module R&R video

Shift selector R&R video

Frame 5-

Header:

Out-Of-Chassis Procedures

Main Text:

Out-of-chassis repair procedures include:

Complete transmission R&R

Input adaptation inspection/replacement

Torque converter R&R

Input seal R&R

Graphics:

Rotating images highlighting each procedure as it is read (taken from TC10 PM shoot)

Links:

Transmission R&R video

Input adaptation inspection/replacement video

Torque converter R&R video

Input seal R&R video

Frame 6-

Header:

Control Module Overhaul

Main Text:

The control module consists of the following:

Turbine speed sensor

Range output speed sensor

Solenoid valve body assembly

Main mod valve body assembly

Clutch valve body assembly

Shift valve body assembly

All of these components can be overhauled separately or as an entire unit.

Graphics:

Start with fully assembled control module then morph to exploded view showing above listed components with interactive roll-overs calling out each one

Links:

Control module disassembly video

Solenoid valve body overhaul video

Main mod valve body overhaul video

Clutch valve body overhaul video

Shift valve body overhaul video

PDF containing line art graphics of:

- Control valve body

- Turbine speed sensor

- Output speed sensor

- Range output speed sensor

- Solenoid valve body

- Main mod valve body

- Clutch valve body

- Shift valve body

Exploded view still image of control valve body with callouts

Frame 7- (NOT SURE IF THIS SLIDE IS RELEVANT IN THIS MODULE AS ALL OF THIS WILL BE COVERED IN THE DIAGNOSTICS MODULE)

Header:

Diagnostic Procedures Overview

Main Text:

TC10 shift selector diagnostics procedures include:

Retrieval and clearing of Diagnostic Trouble Codes (DTCs)

Accessing Oil Level Sensor (OLS) operations

Enabling/disabling prognostics package

Retrieving prognostics information

Resetting prognostics information

Allison DOC™ for PC diagnostic procedures include:

Pressure checks using Clutch Test Action Request in combination with TC10 pressure taps

Clear All Shift Adaptive Data Action Request

Reset Auto-Detect Oil Level Sensor Action Request

Reset SAE J1939 Source Address Information Action Request

Reviewing and resetting prognostics data

Any other DOC Action Requests specific to TC10 product TBD w/Stacy

Allison Calibration Configuration Tool (ACCT) Application.

Extranet application used to create Allison 5th Generation Controls TCM calibrations

Functionality is similar to eCSS, which will be maintained for 4th Gen Controls calibrations

ACCT functionality is addressed in a separate E-Learn module

All of these procedures are addressed in-depth in the TC10 Diagnostics and TC10 DOC Updates modules.

They are listed here for overview purposes only

Graphics:

Rotate through images of shift selectors in diagnostics mode, DOC screen shots and ACCT screen shots

Links:

None

Frame 8-

Header:

Available Publications

Main Text:

Publications available at TC10 SOP include:

Principles of Operation	(PO7115EN)

Troubleshooting Manual (TS7116EN)

Mechanic’s Tips (MT7119EN)

Operator’s Manual (OM7118)

TC10 Service Information Letters (SILs)

Graphics:

Rotating images highlighting each publication cover as it is read

Links:

TC10 Mechanic’s Tips

TC10 Operator’s Manual

TC10 SIL Index Page